MCNAV System & Software Architecture (SSA)

Team MCNAV

| Carnegie Mellon University. |  | Technical Paper |  | Rev: 1.1 |
| --- | --- | --- | --- | --- |
| 17614 – Engineering Software Intensive Systems Carnegie Mellon University Summer ‘05 |  |  |  |  |
| DUE DATE: | 22 July, 2005 |  |  |  |
| TITLE: | DARPA Lunar Grand Challenge 2005 – MBARS |  |  |  |
| AUTHOR(s): | Team MCNAV | DATE: | 22 July, 2005 |  |
| REVIEWED BY |  | DATE: |  |  |
| APPROVED BY: |  | DATE: |  |  |
| ABSTRACT: |  |  |  |  |

Revision History

| Rev | Date | Control No. | Author | Change Comment |
| --- | --- | --- | --- | --- |
| 0.1 | 07/07/2005 | 1 | Changsun, Song | *Create* Template |
| 0.1 | 07/11/2005 | 2 | Gary C. Ackley | *Modify & Refine* Template *Add* Section 3.1,3.2,3.3,3.4 |
| 0.1 | 07/11/2005 | 3 | Changsun, Song | *Modify* Template |
| 0.1 | 07/11/2005 | 4 | Changsun, Song | *Add* subsystem architecture (very draft) |
| 0.1 | 07/14/2005 | 5 | Changsun, Song | *Add* parts from each team |
| 0.1 | 07/14/2005 | 6 | Changsun, Song | *Add* sequence diagrams |
| 1.0 | 07/14/2005 | 1 | Changsun,Song | *Base Line* version 1.0 |
| 1.1 | 07/22/2005 | 2 | Changsun,Song | *Modify* block diagrams, element catalogs, and system modes |

# Document Overview

## Purpose

The objectives of a system and software architecture document are to formally document the architecture including:

- An overview of the system and software architectural drivers

- An overview of the system and software’s architecture

- Relevant views of the system and software’s architecture

- How the system and software architecture will support the achievement of the architectural drivers

- Ancillary information about the system and software architecture

## Definitions

Table 1 – MBARS Terms and Acronyms

| Acronym | Translation | Definition |
| --- | --- | --- |
| DARPA |  | Defense Advanced Research Projects Agency |
| MBARS |  | Moon Based Autonomous Robot System |
| MCNAV |  | Moon Circum-Navigating Autonomous Vehicle |
| OCD |  | Operational Concept Document |
| E-Stop |  | Wireless Emergency Stop Units |
| RTG |  | Radioisotope Thermocouple Generator |

## References

Table 2 – References for Operational Concept Document

| ID | Reference |
| --- | --- |
|  | System Architecture Template, http://www.donald-firesmith.com/index.html |

# System Overview

## System Background and Purpose

In order to accelerate technology development for a Moon Based Autonomous Robot System (MBARS), DARPA has initiated the Lunar Grand Challenge 2005. Carnegie Mellon University’s MCNAV Team (ESIS 2005 - 17614) has taken up the challenge to design a robotics lunar system in accordance with the challenge requirements provided by DARPA.

The lunar environment is extreme. With no atmosphere, the vehicle electronics will be exposed to solar particle events that could literally fry them. Intense ultraviolet radiation will degrade materials. Temperatures on the lunar surface range from minus 170 to plus 110 degrees centigrade depending on whether exposed or shaded from solar radiation.  The challenge is to complete the race route, a total distance of approximately 2826 kilometers (referred to System Assumption) in the least amount of time. The course must be completed in no more than 7 days. The MCNAV must be capable of surviving these conditions and be able to adjust to unforeseen hazards on the lunar surface.

## Architectural Drivers

### Functional Requirements

- The vehicle shall demonstrate fully autonomous behavior at all times during the pre-launch qualifying event and the lunar circumnavigation event.

- The vehicle propulsion and steering shall be through contact with the ground.

- No classified data or devices shall be used in preparation or during the lunar circumnavigation.

- The vehicle shall be equipped with a wireless ESTOP unit supplied by DARPA.

- The vehicle shall not emit or receive any communication other than those used for vehicle tracking by DARPA.

- The system shall be capable of completing a Magellan path to circumnavigate the moon in less than 7 earth days.

### Quality Attributes

### Quality Attribute scenarios are used to specify quality requires for the MCNAV001 system.  The utility tree is presented as a graph with 3 levels as follows

- Quality Attribute,

- Attribute Concern,

- Scenario

- Evaluation - shows the importance as defined by the client and difficulty of implementation per the team evaluation, both on a scale of High, Medium and Low

Table 3 – Quality Attribute Tree

| Quality Attribute | Attribute Refinement | Scenario | Evaluation |
| --- | --- | --- | --- |
| Performance | Power Output | Power generation system shall be capable of providing an output of 780 watts at the peak load of mobility, sensing, navigation and communication. | (H,M) |
|  | Ability to launch to moon | The size of the vehicle, packed for transport, shall be small enough to be accommodated by a rocket payload bay. | (H,M) |
|  |  | The weight of the vehicle shall not exceed 5000 pounds on earth. | (H,M) |
| Modifiability | Size | The size of the vehicle, packed for tarnspor, shall be small enough to be accomodated by a rocket payload bay. | (M, L) |
|  | Complexity | Power generation system requiring supporting equipment increases failure points and weight. It is also harder to modify | (M, M) |
| Availability | HW Failover | Failover to a backup RTG unit failure shall occur in under 10 ms. (e.g. max speed, reduced sensor scan rate) | (L, H) |
|  |  | Power cut-off to a failed H/W sub-component shall occur in under 10 ms to prevent system overload. | (M, H) |
|  | Duration | Vehicle must carry sufficient fuel to last for the duration of the race | (H, H) |
| Reliability | Accuracy of the mobility system | The navigation subsystem relies in the mobility subsystem. If the mobility system is not accurate enough, the racer won't be able to follow the calculated path. In fact, in the worst scenario, the vehicle may end up navigating trough dangerous terrains if the mobility system is not reliable | (H, M) |
|  | Accuracy of the sensing system | The navigation subsystem relies in the sensing subsystem. If the sensing capabilities of the system are not highly accurate, the racer won't be able to follow a calculated path. In fact, in the worst scenario, the vehicle may end up navigating trough dangerous terrains if the sensing system is not reliable | (H, H) |
|  | Accuracy of operation | The power management system monitors the amount of fuel and estimate the distance covered by the remaining fuel. Since the system can change the power mode based on this result, which can affect all other subsystems, the operation shall be accurate. This system allows estimation errors of less than 10 percent. | (H, H) |
|  | Accuracy of sensing | Environment sensing shall detect positive obstacles which are less than 20 meters away and in the vehicle path 100% of the time under all lunar operating conditions. | (H,M) |

# System and Software Architecture

## System Composition

The MCNAV001 system consists of two major components; the facilities to generate a route prior to vehicle launch, and the lunar rover itself.  Each of these consists of several subsystems, which have dependencies illustrated as follows:

Figure 3-1 MCNAV001 Subsystem Dependencies

Table 4 Element Catalog for MCNAV001 Subsystem Dependencies Diagram

|  | Subsystem | Description |
| --- | --- | --- |
| Offline Pre-Processing | Lunar Model | A mathematical representation of the lunar environment that the lunar rover will traverse comprised of elevation and hazard data gathered prior to launch |
|  | Pre-launch Route Generation | Facilities to compute the optimal route, based on the prior data in the lunar model |
| On-Board Lunar Rover | Power System | - The self-contained onboard electrical power generator for the vehicle during operation - Distribute the generated power into all the subsystems |
|  | Mobility System | - The wheels and electric motors capable of moving the vehicle from one location to another - Also provides a dynamic braking system |
|  | Navigation System | The onboard analysis of vehicle trajectory during vehicle operation, and the ability to make decisions to alter the vehicle trajectories to either avoid obstacles, or maintain the optimal pre-computed route |
|  | Environmental Sensing System | The ability to sense the local environment near the vehicle, including the detection of obstacles and other hazards |

## Hardware Components

A representation of the MCNAV system hardware is as follows:

Figure 3-2 MCNAV001 Hardware Design

## Software Components

The system is composed with four main subsystems including environmental sensing manager, mobility controller, navigation manager, and power distribution manager. They communicate with each other via a 1 Gbps Ethernet, and communicate with corresponding devices via CAN bus. This communication method needs to be considered when designing connectors, so this consideration is reflected into the following software block diagram.

Figure 3-3 Software Block Diagram

## Subsystem Description

### Power System

Figure 3-4 Power System

Table 5 Element Catalog for Power Subsystem Diagram

| Component | Description |
| --- | --- |
| Distribution Controller | Distribution controller collects power usage feedback from each sub system. The feedback data is profiled and used for failure detection. This module interfaces the power distribution circuit to cut off power supply to sub-system on detection of short-circuit failure or E-Stop command. This module also collects data from RTG monitoring sensor. Upon detection of a RTG failure, a command signal is generated to each subsystem to activate default power saving operational mode. |
| Ambient Controller | Ambient controller monitors the vehicle internal environment temperature and sets the target temperature for thermal controller. |
| Emergency Manager | Emergency Manager relays the wireless E-Stop system control signal to the Distribution Controller Module for appropriate power delivery control. |

### Mobility System

Figure 3-5 Mobility System

Table 6 Element Catalog for Mobility System

| Component | Description |
| --- | --- |
| Power Constraints Manager | The Power Constraints Manager distributes power among components and ensures enough power is provided to necessary components for McNAV to finish the race safely by reducing power to components that aren’t being used, the system can save the power and reduce peak power demand on the power system |
| Direction Controller | The Direction Controller determines the direction based on the path information from the Navigation Manager. The determination of the direction is calculated from the path information. With the calculated information, the Direction Controller controls the actuators for each wheel through the Wheel Motor Interface. |
| Velocity Controller | The Velocity Controller determines the velocity based on the path information from the Navigation Manager. The velocity information is passed to the Direction Controller, as well as the Brake Interface so as to control the velocity of the vehicle. |
| PathInfo Converter | The PathInfo Converter converts the raw information passed by the Navigation system so that the Direction Controller and Velocity Controller can use the information. |

### Environment Sensing System

Figure 3-6 Environment Sensing System

Table 7 Element Catalog for Environment Sensing System

| Component | Description |
| --- | --- |
| Scanning LADAR Sensor Driver | The Scanning LADAR Sensor Driver controls the LADAR sensors so that the Object Detector component can use the information from the sensors |
| FLIR Sensor Driver | The FLIR Sensor Driver controls the FLIR sensors so that the Object Detector component can use the information from sensors. |
| Object Detector | The Object Detector detects the obstacles around the moving locomotive using the information from the Scanning LADAR Sensor Driver and the FLIR Sensor Driver. The information about the obstacles is passed into the Navigation system. |
| Power Constraints Manager | The Power Constraints Manager distributes power among components and ensures enough power is provided to necessary components for McNAV to finish the race safely by reducing power to components that aren't being used, the system can save the power and reduce peak power demand on the power system |

### Navigation System

The following diagram shows the component and connector view for the Navigation subsystem.

Figure 3-7 Navigation System

Table 8 Element Catalog for Environment Sensing System

| Component | Description |
| --- | --- |
| Power Constraints Manager | The Power Constraints Manager distributes power among components and ensures enough power is provided to necessary components for McNAV001 to finish the race safely by reducing power to components that aren't being used, the system can save the power and reduce peak power demand on the power system |
| PathFactory | The Path Factory computes the optimal path for the vehicle to reach the next waypoint in the pre-computed route. It evaluates each possible arc is in terms of least cost to the goal. |
| Navigation Controller | Navigation Controller receives a path from the Path Factory, and translates this to speed and curvature commands to send to the Mobility Controller. |
| Object Avoider | The obstacle avoider receives environment sensing data to detect obstacles and uses an algorithm to avoid the vehicle from moving into unknown terrain and therefore will recommend good steering commands to the selection module, and vetoes dangerous commands. |
| Lunar World Model | The Lunar World Model holds the operating environment representation data, and is constantly updated with sensed data captured during the race and is used for avoidance of previously undetected obstacles and real-time path planning. |
| Emergency Manager | Emergency Manager relays the wireless E-Stop system control signal to the Distribution Controller Module for appropriate power delivery control. |

## System Modes

The MCNAV001 system will support the following modes:

Table 9 System Modes

| Mode | Description |
| --- | --- |
| OFF | The initial state of the lunar rover is OFF. While in this state the dynamic braking system is engaged, the computer systems are off, and RTG is not providing power. The lunar rover is in a transportable state only while OFF. The lunar rover will automatically begin shutdown to OFF upon receiving a DISABLE command from the ESTOP device. |
| INIT | This lunar rover will automatically transition to INIT after activating the thermocouple power generator. After RTG is activated, the rover will apply power to the processing subsystem. On power-up, the processing subsystem will begin the built-in self-test of all onboard computing systems, sensors and actuators. Following successful system initialization, the system will automatically transition to the PAUSED state. If initialization fails, the system will automatically transition to the ERROR state. |
| IDLE | The lunar rover will automatically transition to IDLE after successful system initialization. While in the IDLE state, RTG is supplying power to the system, the dynamic braking system is engaged, and the system is awaiting a signal to begin executing its pre-computed route. The system will remain in the IDLE state for as long as the lunar rover does not receive a RUN command from the DARPA provided ESTOP device. While in the IDLE state, an external computer system may be attached to the lunar rover’s onboard computer network for the purposes of upload a lunar model and navigation route to the onboard navigation computer, or for performing system diagnostics. |
| ACTIVE | The lunar rover will transition to ACTIVE after receiving a RUN signal from the DARPA provided ESTOP device. After receiving the ESTOP signal, the lunar rover will wait 5 minutes before resuming the lunar circumnavigation route. The lunar rover will automatically transition to IDLE upon receiving a PAUSE command from the ESTOP device. Upon successful completion of the pre-computer lunar circumnavigation route, the lunar rover will automatically transition to IDLE. |
| ERROR | The lunar rover will automatically transition to an ERROR state if the built-in self-test fails or a system failure is detected during system operation. From this state, the system must be turned off to reset. While in the ERROR state the vehicle will stop with the dynamic braking system engaged. From this state, an external computer system may be attached to the lunar rover's onboard computer network for the purposes of system diagnostics while on earth. From this state, the system must be turned off and re-started to reset. Error mode will be disabled prior to delivery to DARPA for launch. |

## Sequence Diagrams

The sequence diagrams show the interaction of the subsystems of the McNAV001 system. The sequence of system transitions is illustrated as follows

### Turn On

Figure 3-8 Turn On Sequences

### Turn Off

Figure 3-9 Turn Off Sequences

### ESTOP RUN

Figure 3-10 ESTOP Run Sequences

### ESTOP PAUSE

Figure 3-11 ESTOP PAUSE Sequences

### ESTOP DISABLE

Figure 3-12 ESTOP DISABLE Sequences