MCNAV Reliability Analysis

| Carnegie Mellon University. |  | 12121212 |  | Rev: 1.0 |
| --- | --- | --- | --- | --- |
| 17614 - Engineering Software Intensive Systems Carnegie Mellon University Summer ‘05 |  |  |  |  |
| DUE DATE: | 01 July, 2005 |  |  |  |
| TITLE: | DARPA Lunar Grand Challenge 2005 - MBARS |  |  |  |
| AUTHOR(s): | Team MCNAV | DATE: | 01 July, 2005 |  |
| REVIEWED BY | Team MCNAV | DATE: |  |  |
| APPROVED BY: | Team MCNAV | DATE: |  |  |
| ABSTRACT: | This document provides an analysis of the reliability of the proposed MCNAV001 system – with particular emphasis on the probability that the system will complete the challenge. |  |  |  |

Revision History

| Rev | Date | Control # | Author | Change Comment |
| --- | --- | --- | --- | --- |
| V0.1 | 07-05-2005 | 1112 | Gary Ackley | Create the template |
| V0.2 | 07-14-2005 | 1112 | Gary Ackley | Update with customer comments Complete FMECA Complete FTA |
| V1.0 | 07-16-2005 | 1112 | Gary Ackley | Incorporate Power updates Incorporate Mobility updates Incorporate Sensing update |

Table of Contents

1.	Introduction	3

1.1	Definitions	3

2.	System Overview	3

3.	System Reliability	3

3.1	Failure Mode, Effects and Criticality Analysis [FMECA]	3

3.1.1	Define System Requirements	3

3.1.2	Functional Analysis	4

3.1.3	Requirements Allocation	4

3.1.4	Failure Modes	4

3.1.5	Causes of Failure	4

3.1.6	Failure Effects, Severity, Frequency, Detection Means and Probability	6

3.1.7	Recommendation	8

3.2	Reliability Prediction	9

3.2.1	Power Generation	10

3.2.2	Environment Sensing	10

3.2.3	Processing	11

3.2.4	Navigation	12

3.2.5	Mobility Locomotion	12

3.3	Fault-Tree Analysis [FTA]	13

List of Figures

Figure 3-1 Ishikawa Cause & Effect [Fishbone] diagram	6

Figure 3-2 MCNAV001 Reliability Block Diagram	10

Figure 3-3 Environment Sensing Reliability Block Diagram	11

Figure 3-5 Fault Tree Analysis	13

List of Tables

Table 3-1 Failure Severity and Frequency Values	6

Table 3-2 Failure Detection Probability Values	7

Table 3-3 FMECA Summary	8

# Introduction

The purpose of the Reliability Analysis report is to present an analysis of the probability that the MCNAV001 system will successfully complete the challenge.

## Definitions

| Acronym | Definition |
| --- | --- |
| DARPA | Defense Advanced Research Projects Agency |
| MBARS | Moon Based Autonomous Robot System |
| MCNAV | Moon Circum-Navigating Autonomous Vehicle |
| FMECA | Failure Mode, Effects and Criticality Analysis |
| FTA | Fault-Tree Analysis |
| LADAR | Laser Detection and Ranging |
| FLIR | Forward Looking Infra-Red |

# System Overview

In order to accelerate technology development for a Moon Based Autonomous Robot System (MBARS), DARPA has initiated the Lunar Grand Challenge 2005. Carnegie Mellon University’s MCNAV Team (ESIS 2005 - 17614) has taken up the challenge to design a robot system according to DARPA contest.

The challenge is to complete the race route, a total distance of approximately 2826 kilometers (referred to System Assumption) in the least amount of time. The course must be completed in no more than 7 days. The MCNAV must be capable of surviving these conditions and be able to adjust to unforeseen hazards on the lunar surface.

# System Reliability

The analysis techniques of interest in this report include FMECA and FTA.  Each of these is described in subsequent sections.

## Failure Mode, Effects and Criticality Analysis [FMECA]

The FMECA is a technique that identifies weaknesses in a product or process. To complete this analysis, a Failure Mode, Effects and Criticality Analysis [FMECA] is as follows:

### Define System Requirements

Describe the system in question, the expected outcomes, and the relevant technical performance measures [TPMs].

In order for the vehicle to accomplish the mission within the allowable time the vehicle must complete the course in at least 7 days.  Thus, the MFBF rating is 7 days or 168 hours.  This requirement becomes the technical performance measure [TPM] for correct system operation.  Due to the serial nature of the system, this TPM must be applied to every MCNAV subsystem.

### Functional Analysis

Define the system in functional terms.

See the System and Software Architecture document for a description of the allocation of requirements to subsystems.

### Requirements Allocation

Top-down breakout of system-level requirements.

See the System Capability Requirements & Qualification Method document for a description of the allocation of requirements to subsystems.

### Failure Modes

An examination of the system block diagram illustrates the following possible failures:

- Chassis Failure = Loss of chassis integrity during challenge

- Power Failure = Loss of electrical power during challenge

- Mobility failure = Inability for vehicle to move across surface

- Navigation Failure = Inability for robot to compute route over lunar surface

- Environment Sensing Failure = Inability to sense the local environment within the vicinity of the lunar vehicle.

- Processing Failure = Inability of computing system to function

### Causes of Failure

Analyze the product to determine the actual cause(s) responsible for failure.  An Ishikawa “cause-and-effect” diagram may prove to be effective.

The failure modes of the system are represented in a simplified representation of an Ishikawa (fish-bone) diagram as follows:

- Chassis Failure

  - Due to vibration and shock during delivery to challenge site

  - Due to vibration and shock according during operation

- Power Failure

  - Due to exhausted fuel supply

  - Due to a defect in one or more fuel pellets

  - Due to manufacturing defect in generator

  - Due to wiring connection failure

    - Due to vibration & shock during delivery

    - Due to vibration & shock during operation

- Mobility Failure

  - Due to wheel axel failure

  - Due to actuator (motor) failure

    - Due to manufacturing defect

    - Due to vibration & shock during delivery

- Navigation Failure

  - Due to environment sensing failure

  - Due to loss of position estimation

    - Due to loss of signal from Loran-C

    - Due to loss of information from wheel encoders

      - Due to wiring failure

        - Due to vibration & shock during delivery

        - Due to vibration & shock during operation

- Environment Sensing Failure

  - Due to thermal control failure

  - Due to wiring failure

    - Due to vibration & shock during delivery

    - Due to vibration & shock during operation

  - Due to sensor failure

    - Due to vibration & shock during delivery

    - Due to vibration & shock during operation

    - Due to manufacturing defect

- Processing Failure

  - Due to thermal control failure

  - Due to radiation damage

  - Due to equipment fault

Figure 3-1 Ishikawa Cause & Effect [Fishbone] diagram

### Failure Effects, Severity, Frequency, Detection Means and Probability

Consider the effect of the failure on other elements at same or higher level in the system, and from the system overall.  Identify the seriousness of the effect or impact of a particular failure.  Address the frequency of the occurrence of each individual failure mode.

The severity and frequency of the failures is indicated as follows:

| Value | Severity | Frequency |
| --- | --- | --- |
| 1 | Minor | Remote |
| 2-3 | Low | Low |
| 4-6 | Moderate | Moderate |
| 7-8 | High | High |
| 9-10 | Very-high | Very-high |

Table 3-1 Failure Severity and Frequency Values

Identify the process controls which may detect the failures.  What is the probability that process controls, design features, verification procedures, etc. detect the potential failures in time to prevent a major system catastrophe?

The severity and frequency of the failures is indicated as follows:

| Value | Detection Probability |
| --- | --- |
| 1 | Very-high |
| 2-3 | High |
| 4-6 | Moderate |
| 7-8 | Low |
| 9 | Remote |
| 10 | Absolute certainty of non-detection |

Table 3-2 Failure Detection Probability Values

Identify the critical aspects of the system design in terms of severity, frequency and probability of detection, expressed as a risk priority number (RPN).

The criticality of the risks is expressed as a risk priority number (RPN), computed as:

RPN=(severity rating)(frequency rating)(detection probability)

A table summarizing the FCEMA analysis is provided in Table 3-3 FMECA Summary

Table 3-3 FMECA Summary

### Recommendation

Based on the above analysis, the following architectural features or additional process steps are recommended:

- The vehicle shall undergo shake testing prior to final delivery

- The vehicle shall undergo radiation exposure testing prior to final delivery

- The vehicle shall undergo temperature testing prior to final delivery

- The vehicle shall perform built-in-self tests prior to operation

- The computing system shall employ redundant watchdogs such that in the event that any one computing system fails, it shall be restarted.

- Multiple wheel motors shall be used in the vehicle design to ensure redundancy of locomotive ability

- Multiple wheel encoders shall be used for position estimation through odometry

- All sensitive electronic components shall be shielded from electromagnetic radiation

- All sensitive electronic components (particularly sensors) shall have temperature controls to ensure operation within sensor environmental limits

## Reliability Prediction

Compare the predicted values of MTBM/MTBF against the requirements – looking for incompatibilities or areas for improvement.

We want a racer reliability of one in one hundred chance of racer failure during the race.  Assuming the racer takes the full 7 days (168 hour) to complete the course, this implies that the reliability (R) and the mean-time between failure (MTBF) for the racer is as follows:

R=1−failuresattempts=1−1100=0.99R=e−tMTBF⇒MTBF=−tln(R)=−168hln(0.99)=16715.8h

This corresponds to nearly 2 years of continuous fault-free operation.

A chart illustrating various reliability rates is as follows:

Note that even a 50% chance of winning requires that the racer be capable of operating for over 242 hours – which is nearly 1.5 times the duration of the challenge.

At a high level from the point of view of system reliability, the MCNAV001 system is represented as a series network of 5 components as follows:

Figure 3-2 MCNAV001 Reliability Block Diagram

Each of these subsystems represents a single point of failure.  Should any one component in the above series fail, then the lunar rover will not meet the challenge.  (Note: There are additional elements of the system, such as Chassis, are beyond the scope of this document, and are therefore not included in this analysis.)

Allocating the above reliability equally between the four components yields:

Rtotal=RpowerRsensingRprocessingRnavigationRmobility=Rcomponentl5∴Rcomponentl=5Rtotal=50.99=0.998R=e−tMTBF⇒MTBF=−tln(R)=−168hln(0.998)=84415.5h

Thus each component must be able to operate continuously without fault for over 9.5 years.  If this is possible, then the system will have a 99 out of 100 chance of not failing during the 7 day mission.

Each of the subsystems have been analyzed, and then combined to get a picture of the reliability of the system as a whole.

### Power Generation

The power generation system is composed of three RTG packages in a parallel network.  The RTG uses pu-238 with a half life of 87 years.  This proper ensure that an RTG can still supply 90% of initial max power output after 5 years of operation.  Both of these number well exceeded the 7 days (168 hours) of operation MBTF demand by the mission, therefore, the power system reliability rests mainly in manufacturing defect and inter-subsystem power distribution connections (RGT does not contain moving parts).  To achieve a component reliability of 0.998, both RTG manufacturing and fuel pellet manufacturing reliability must achieve 0.999(1/3) (3 RTG on board) and 0.999(1/216) (72 pellets per RTG) respectively as shown below.

R=RRTG3Rpellet216=0.999×0.999=0.998

### Environment Sensing

Environment sensing consists of a pair of scanning LADAR sensors as well as a stereo FLIR sensor, all of which provide sensor data into an Object Detection software component.  While it is not required that all the sensors to be working for obstacle detection to occur, several sensors are preferred to permit a greater probability that obstacles within the vehicle trajectory will be detected.  A block diagram of the environment sensing subsystem is as follows:

Figure 3-3 Environment Sensing Reliability Block Diagram

A Laser Detection and Range finding [LADAR] sensor is a highly accurate instrument for measuring distances to obstacles to within a few centimeters.  However, due to the nature of LASER emission, the sensor is only capable of measuring distance to a single point at a time.  To enable the sensor to scan a 2-dimensional area requires a spinning mirror, and an oscillating actuator.  The mirror deflects the beam along one axis, while the oscillating mechanism rotates the entire instrument along the other axis.  Due to the accuracy of the LADAR, mechanical complexity of the scanning mechanisms, the lunar temperature extremes and radiation hazards, a single scanning LADAR has an estimated reliability of 0.80 on the lunar surface.

A Forward Looking Infra-Red [FLIR] sensor contains an array of micro-bolometers on a chip and is used to measure temperature differences.  Similar to visible-light camera technology, a FLIR is capable of sensing an entire 2-dimensional field of view at relatively high-resolution.  To achieve depth-perception requires two or more sensors be used to obtain multiple-images.  Distances to obstacles are triangulated using differences between simultaneously captured images given the known distance between the sensors.  Due to the mechanical simplicity of FLIR sensors, lunar temperature extremes and radiation hazards, a single FLIR sensor has an estimated reliability of 0.90

Obstacle detection is the act of processing the raw sensor data, and computing an estimation of obstacles which are in the vicinity of the sensors.  Obstacle detection is a probabilistic exercise using Kalman filters which compensate for sensor noise.  Due to lunar temperature extremes and radiation hazards to the processing elements, and the probabilistic behavior of the Kalman filters, obstacle detection has an estimated reliability of 0.95.

The combined reliability of the environment sensing subsystem is as follows:

Rsensing=1−1−RFLIR21−RLADAR2RObstacleDetection=0.94278

### Processing

### Navigation

The navigation system reliability is composed of a positioning and a path planning component with a series network relationship, given that both components must operate properly for the correct system functioning. The path planning component is a single software component and the positioning component reliability is analyzed below. The navigation reliability is expressed as:

R=(Rpos)(Rpp)=.999∗.999=.998

Figure 3-3-5-2 Navigation Reliability Block Diagram

#### Positioning

The positioning system consists of Odometry System and Loran-C receiver used in combination to reduce the error accumulation over the length of the race. Both systems provide positioning as parallel networks. Their positioning reliability is as:

R=Ro+Rl−(Ro)(Rl)=.999+.87−(.999∗.87)=.99987

Figure 3-3-5-1 Position Estimation Reliability Block Diagram

### Mobility Locomotion

Mobility / Locomotion system reliability consists of a command perception and a locomotion control component with a series network relationship.

Figure 3-3 Mobility / Locomotion Reliability Block Diagram

The mobility / locomotion reliability is expressed as:

The command perception component is out of our scope, so we will focus on the locomotion control component.

#### Locomotion control

Locomotion control consists of Movement component that includes motor and brakes and Direction component that includes steering and actuators.

Figure 3-4 Mobility Control Reliability Block Diagram

Both components provide positioning as parallel networks: Their reliability is as follows:

## Fault-Tree Analysis [FTA]

Provide a graphical enumeration and analysis of different ways a failure can occur, with its probability of occurrence.

A summary of the fault tree analysis is as follows:

Figure 3-4 Fault Tree Analysis

Note: It is difficult in the FTA to avoid circular dependencies.

For example, A Processing failure may occur because of a power failure, because of a Power Management failure, because of a Processing failure.  Further, nearly all the systems rely on power, and processing (which is reliant on power).

Systems Engineering and Analysis, Third Edition; Blanchard; © 1998 Prentice-Hall – page 347-348

Systems Engineering and Analysis, Third Edition; Blanchard; © 1998 Prentice-Hall – page 353