MCNAV System Test Plan

Team MCNAV

| Carnegie Mellon University. |  | Technical Paper |  | Rev: 2.1 |
| --- | --- | --- | --- | --- |
| 17614 - Engineering Software Intensive Systems Carnegie Mellon University Summer ‘05 |  |  |  |  |
| RELEASE DATE: | 24 July, 2005 |  |  |  |
| TITLE: | DARPA Lunar Grand Challenge 2005 – MBARS |  |  |  |
| AUTHOR(s): | Team MCNAV | DATE: | July 24, 2005 |  |
| REVIEWED BY | Team MCNAV | DATE: | July 24, 2005 |  |
| APPROVED BY: | Team MCNAV | DATE: |  |  |
| ABSTRACT: | This document describes system test plan for the proposed MCNAV01 system, a competitor in the DARPA Lunar Grand Challenge 2005, in terms satisfying MBARS requirements outlined by DARPA. |  |  |  |

Revision History

| Rev | Date | Control No. | Author | Change Comment |
| --- | --- | --- | --- | --- |
| V0 | 07-06-2005 | 0 | Carl Wang | *Create* the template |
|  | 07-12-2005 | 1 | Carl Wang | Consolidate template feedback from GHC, GSL, GCA, and Sam. |
|  | 07-15-2005 | 2 | Carl Wang | Consolidate input from GSL, GCA, and JWB for 1st draft release |
| V1 | 07-19-2005 | 0 | Carl Wang | Consolidate input from MYP and OAS for a completed 1st draft release |
|  | 07-19-2005 | 1 | Ghislaine Hernandez | Reviewed and reformatted to fit 10 pages |
| V2 | 07-24-2005 | 0 | Carl Wang | Incorporate input from Gary for a 2nd draft release |
|  |  |  | Carl Wang | Consolidate input from GHC, |

Table of Contents

1.	Document Overview	2

2.	Scope and Objectives	4

3.	System Test Resources	7

4.	Problem Evaluation Process	7

5.	System Test Case (STC)	8

# Document Overview

## Project Objective

DARPA initiated the Lunar Grand Challenge 2005 to accelerate technology development for a Moon Based Autonomous Robot System (MBARS).  Carnegie Mellon University’s MCNAV Team (ESIS 2005 - 17614) designed the MCNAV01 (self propelling robotics lunar system) to challenge the lunar circumnavigation race as set forth by DARPA.

MCNAV01 is designed to operate in the harsh lunar environment. The vehicle has to withstand the extreme temperature from minus 170 to plus 110 degrees centigrade depending on whether exposed or shaded from solar radiation and exposure to solar particle events.  The challenge is to complete the race route, a total distance of approximately 2826 kilometers (referred to System Assumption) within 7 earth days. The MCNAV01 must be capable of surviving these conditions and be able to adjust to unforeseen hazards on the lunar surface autonomously during the course of race.

### DARPA Requirements

System Constraints:

The system is constrained by the natural environment as well as grand challenge (race) rule.  MCNAV01 must operate in the harsh lunar environment and retains operational function after launching from earth and landing on the moon.  Additional customer rules are described in ESIS Lunar Grand Challenge rules.

System Requirements:

System requirements are described in detail by MCNAV System Capability Requirements and Qualification Method document.

## System Description

MCNAV01 is composed of advanced sub-systems operating cooperatively to satisfy requirements as set off by DARPA for the grand challenge.  In addition to mobility (locomotion) and power system as all moving vehicle contains, MCNAV01 is installed with advanced environment sensing and navigation systems that are responsible for autonomously providing driving direction and real-time obstacle avoidance without manual assistance.  The vehicle is installed with a communication system to provide remote location identification to NASA.  The on board processing system is capable of monitoring power distribution and controlling operation of subsystems in response to emergency (power or component failure accordingly) to accomplish the race.

## Purpose of System Test Plan

The purpose of the System Test Plan is to specify system verification and validation tests for MCNAV01 system.  The system test plan shall address the validation tests to demonstrate MCNAV01 system satisfaction against customer (DARPA) requirements by series of function validation demonstration.  The system test plan also address the verification tests to be performed prior to system launching on mission to moon to demonstrate system readiness for the mission by series of built in self-test and function verification demonstration.  The plan addresses the system operational evaluation test environment setup, supporting facility, personnel training, and procedures to be followed for system test.

## Definitions

Table 1 – MBARS Terms and Acronyms

| Acronym | Translation | Definition |
| --- | --- | --- |
| DARPA |  | Defense Advanced Research Projects Agency |
| MBARS |  | Moon Based Autonomous Robot System |
| MCNAV |  | Moon Circum Navigating Autonomous Vehicle |
| NASA |  | National Aeronautics and Space Administration |
| CCB |  | Configuration Control Board |
| ECP |  | Engineering Change Proposal |

## System Assumption

Table 2 – Key Assumptions from DARPA Lunar Challenge Rules

| ID | Assumption |
| --- | --- |
|  | DARPA will install 2 Loran-C transmitters on the moon |
|  | The race will occur following a Magellan Route above 75th parallel. Therefore, the race will be limited to a length of 2826 km (1757 miles). |
|  | The race will be limited in duration to 1 earth week. Therefore, to cover the race distance the robot will need to travel at an average speed of 4.7 m/s (10.4 mph). |
|  | Electric powered motors shall be used for vehicle locomotion. |
|  | Software updates are permitted up to 1 min prior to starting the lunar circumnavigation trek. |
|  | DARPA will be responsible for delivery of the rover to the lunar surface prior to the race. |

## Quality Assurance Involvement

The Quality Assurance (QA) team is responsible for operational testing to validate and verify that the system is compliant to regulatory & safety requirements.  The QA team with support from the System Installation & Support Team is responsible for operation testing to validate and verify MCNAV01 performs required functions with acceptable reliability to sustain the challenge race.

## References

Table 3 – References for System Test Plan

| ID | Reference |
| --- | --- |
|  | ESIS Lunar Grand challenge rules |
|  | MCNAV System Capability Requirements and Qualification Method document |
|  | SQA Test Plan Sample Template * |
|  | Sample System Test Plan Template * |
|  | FIS Upgrade System Test Plan * |
|  | MCNAV Technical Paper |
|  | MCNAV Operation Concept document |
|  | MCNAV Reliability Analysis document |
|  | System Engineering & Analysis 3rd edition by Blanchard B & Fabrycky W. |

* Hyper-links to online document.

# Scope and Objectives

## Scope of System Functions under test

The system test plan is intended to cover the pre-challenge testing.

    - Inclusions

All system qualification and some subsystem qualification applicable on system level listed in the MCNAV System Capability Requirements and Qualification Method document are required to be verified during system testing unless otherwise noted as exclusion below.

### Exclusions

Table 4 - Excluded System Test Lists

| Requirement | Function | Reason |
| --- | --- | --- |
| General | Development Unit, component, and Integration Testing | Document scope |
| Physical REQ-PC-2 | Vehicle must not damage environment | Requirement too broad. Infinite possible tests. Is implicitly demonstrated by obstacle avoidance and safety regulation compliance. |
| REQ-ER-2 | Vehicle operation under vacuum | Difficulty in accessing necessary environment |
| Environmental REQ-ER-4 | Vehicle damage when exposed to solar radiation | None observable feature. Only selects exposed parts that are radiation proof. |
| Support | Manual maintenance | Manual maintenance is not possible once MCNAV01 is launched from earth. |

## Testing Method (Techniques)

Description of methods used for evaluation

- Examination – Inspection consisting of visual, or auditory, or olfactory, or tactile, or other investigation without the use of special laboratory appliances, or procedures, or supplies and services to determine conformance to the specified requirements which can be determined by such investigations.

- Analysis – Verification of applicable document or test results by established mathematical or technical models or simulations, algorithms, charts, graphs, circuit diagrams, or other scientific principles and procedures to provide evidence that the stated requirement was met.

- Demonstration – Actual operation, adjustment, or reconfiguration of items to provide evidence that the designed functions were accomplished.

- Test (measurement) – Verification and inspection shall generally denote the determination, by technical means, of the properties or elements of items, including functional operation, and involves the application of established scientific principles and procedures.

- Certification – A document that include test results, performance data, analytical data, or vendor document asserting that the item complies with the applicable requirements.

- Implicit – Conformance to the requirement is verified by completion of all other tests in the verification matrix.

## Testing Sequence & Scope

Pre-Challenge System tests are to be carried out in the following sequence with limited scope at each step to establish confidence in system operability while minimizing duplicate testing effort.

Static Testing

- Personnel Test and Evaluation – Tester for each specific system test is responsible for getting a clear objective of the assigned test, vehicle operational instruction, and maintenance & installation manual by examination.

- Software Verification – When applicable, software shall provide self-test capability and when possible, incorporate hardware on board self-test results to provide individual sub-system operational status.  At the minimum, each hardware board shall provide power-up self-test status report.  The system software development & integration team shall demonstrate this capability (see section 5).

- Technical data verification – Outstanding technical data provided by vendor or assumed during the design & development phase that has not been (or unable to) validated at component & integration test shall be tested during the system validation phase.

- Regulatory compliance test – Validation of compliance to local regulatory rules as listed below, must be completed prior to field-testing, either by certification or laboratory tests.

  - Laser Safety Standards (OSHA 29 CFR 1926.54 & TED 1-0.15A Section III)

  - RF Radiation Standards (OSHA 29 CFR 1910.97 & DoD Instruction 6055.11)

- E-Stop system testing – Demonstration of capability to stop the operation of MCNAV01 at a 100% confidence level is required prior to starting field tests.

- Environmental safety testing – The MCNAV01 system uses radioactive substance for fuel.  Data analysis or implicit tests demonstrating proper containment of this hazardous substance in case of an accident (e.g. fire, collision impact, or flood) must be confirmed in laboratory prior to field-testing.

Operational Testing (in field)

- Performance test – The objective of this test is to ensure that MCNAV01 provides acceptable response to satisfy specified performance requirements specified (reference [3]) by demonstration or specific test(s).

- Environmental qualification test – The objective of this test is to verify that MCNAV01 can operate under the harsh lunar environment by implicit testing and certification.

- Stress tests – The objective of this test is to verify MCNAV01 is capable of continuous operation and physical abuse (launch and landing operation) required for the challenge.

- Regression test – Repeat of test steps 1 ~ 9 for resolution of each critical issue identified or steps 7 ~ 9 in different field-test environment.

- Reliability qualification tests – Reliability analysis shall be performed on the system or sub-systems when applicable as outlined in the MCNAV Reliability Analysis Document.

- Pre-Launch Qualification Test (Operational Acceptance Test) – Customer demonstration of MCNAV01 performance for qualification to the grand challenge.

## Testing process

Figure 1 Testing process life-cycle

## Test Approval Criteria

A test step is considered completed upon sign off of the completion criteria by the test controller.

Deliverables

- Completed test log/script

- Completed test results (data) shall be archived to the system evaluation database

- Test problem report (if identified – see section 4)

Completion Criteria

- All processes completed successfully providing the expected qualification results

- Proper resolution of identified problems.

## System Test Entrance/Exit Criteria

### Entrance Criteria

The following is a list of entry criteria that shall be fulfilled before commencement of system tests.  Any deviation from this list shall be approved jointly by the system test controller, the customer representative, and the project manager.

- All development code shall be unit tested.

- All sub-systems shall be tested and verified with on board self-test installed.

- All required hardware, software, and test equipment shall be in place.

- System feature(s) completion list shall be sign-off by system integration team (SIT).

- Generated operations acceptance test checklist shall be signed-off by customer representative.

- All system test responsibilities shall be assigned to specific individual testers.

### Exit Criteria

- All high severity issues (A or B) identified shall be resolved and approved by CCB.

- Outstanding issues shall be identified and captured as a risk and signed-off as acceptable by the customer representative and the CCB.

- Vendor supplied critical operational data shall be validated by test(s) or certification.

- All items on operations acceptance test checklist shall be signed-off by customer representative

# System Test Resources

This section describes additional resources required to support the system testing.

## Hardware

- Transportation for moving MCNAV01 and testing personnel to test site.

- RTG Transportation System – since RTG contains plutonium-238, Nuclear Regulatory Commission (NRC) imposes strict regulations on transportation rule and NASA imposes thermal performance on these devices.

- Personnel room & board facility at test site

- Test result recorders and storage

- (Direct/Remote) Communication Equipment to MCNAV01

- Environmental Test Chambers

- Shake table or alternatively Crane and Weight (to simulate landing/launch impact)

## Software

- Vehicle operational data logging and analysis tools

- Vehicle Simulator

- Vehicle application debug/simulation/upgrade/download tools

- Path planning tool.

## Test Site selection

The operational qualification test shall be conducted in one of the four test locations based on the feature under test.

- Development Office – for vehicle simulation demonstration

- Rocky climax – for obstacle testing.

- Death Valley, CA – for operational testing in high temperature

- Antarctic – for operational testing in low temperature

# Problem Evaluation Process

Evaluation of system issues rely on analysis of operational data collected.

## Problem Reporting

Problems identified during the system test can be a tester (operator) error, procedural error, test configuration error, or actual system function error.  The operator, procedural, and configuration errors are corrected by updating test script and therefore are resolved by the system test (QA) team following procedure outlined in Figure 1. In the case of a system function error, a formal evaluation process by the CCB, as outlined in Figure 2isshall be followed from the problem reporting steofin Figure 1 eren an engineering change proposal (ECP) is created.

### Severity Assignment

Each problem report generated from the system test process shall be assigned a severity level in addition to the test case, data, configuration, and sequence description.  The severity level is assigned as follows:

- "A-mission critical", i.e. can't win the race with this error in the system.

- "B-significant loss of performance", i.e. probably will not win the race.

- "C-nuisance”, i.e. nice to have this error fixed but have no affect on the race outcome.

- "W-watch for recurrence", i.e. intermittent and no observable affect on system operation.

## Problem Evaluation Process

* ref (9)

Figure 2 System Change Control Process

# System Test Case (STC)

Detail description of each test procedure is captured in test script for individual test cases.  Section 5.1 provides a traceability matrix of STC to system requirement as provided by ref 2.  Section 5.2 ~ 5.5 provides a summary of built-in-test (BIT) that are automatically executed by MCNAV01 on system power up to self-validate the operational status of each subsystems.  These BITs are used in system test as well as prior to launch to provide system status.

## System Requirement Traceability Summary

Table 5 – System Test Requirement traceability matrix

| STC-ID | Requirement | Verification Item | Method | Site |
| --- | --- | --- | --- | --- |
|  | REQ-PE-1 | Vehicle operational speed | 4 | 1,2,3,4 |
|  | REQ-PE-2 | Communication with Loran-C (DARPA tracking device) | 4 | 1 |
|  | REQ-PE-2 | Silent vehicle operation (moving without external communication) | 3 | 1,2,3,4 |
|  | REQ-ES-1,2 | Obstacle detection/avoidance | 3 | 1,2,3,4 |
|  | REQ-MB-1 | Vehicle Maneuver Mechanical Response Delay | 2, 4 | 1,2,3,4 |
|  | Performance | Remote software download (data download via satellite) | 3 | 1,3,4 |
|  | REQ-PC-1 | Reliability test (operation duration) | 3, 4 | 2,3,4 |
|  | Physical | Vehicle locomotion status | 1 | 1,2,3,4 |
|  | REQ-PC-3 | Vehicle operational weight | 4 | 1 |
|  | REQ-SA-6 | Vehicle Towing Support | 1 | 1 |
|  | REQ-PC-4 | Visual vehicle identification & operation mode indication | 1, 3 | 1 |
|  | Physical | Impact resistance | 2, 3, 4 | 1 |
|  | REQ-SA-1 | Laser emission | 5 | 1,2,3,4 |
|  | REQ-SA-2 | RF radiation | 5 | 1,2,3,4 |
|  | REQ-SA-3~5 | Emergency stop control | 3 | 1,3,4 |
|  | REQ-PC-2 | Radioactive emission | 4 | 1,2,3,4 |
|  | REQ-ER-1/PC-2 | Autonomous Navigation | 3 | 1,2,3,4 |
|  | REQ-ER-3 | Vehicle operation under extreme temperature affect | 2, 3 | 3,4 |
|  | REQ-ER-5 | Vehicle operation in darkness (without light) | 3 | 1,2,3,4 |
|  | Documentation | Vehicle operation instruction manual | 1 | 1 |
|  | Documentation | Vehicle maintenance and installation instruction | 1 | 1 |

Note: Component validation is limited to sub-component requirements qualified by above system test cases.

  - Mobility/Locomotion

| Testing Target | PEGASUS |
| --- | --- |
| Objective | Tests the suspension of PEGASUS and the elastic loop wheels |
| Function Verified | - The distribution of the vehicle load (weight) to each wheel - The function to control the rigid-step height - The function to control the degree for climbing a slope - The function to control the degree for descending a slope |

| Testing Target | Power Train |
| --- | --- |
| Objective | Tests the motor control and brake system |
| Function Verified | - Ability to make vehicle accelerate to a specific speed - Function to make vehicle decelerate and stop completely - Report abrasion status of brake |

| Testing Target | Steering |
| --- | --- |
| Objective | Tests the steering and actuators |
| Function Verified | - Function to control angle and torque to proceed to a specific direction - Ability to follow steering commands coming from navigation subsystem |

  - Power Generation

| Testing Target | Ambient Temperature Control Subsystem |
| --- | --- |
| Objective | Test the temperature controller |
| Function Verified | - Communication with temperature controller - Status of target ambient temperature control channel - Status of temperature monitoring sensor |

| Testing Target | Distribution Controller |
| --- | --- |
| Objective | To ensure the generated power is distributed to all other subsystems and the controller can accurately monitor the amount of power that is used for each subsystem. |
| Function Verified | - The status that indicates the amount of generated power (RTG Status) - The status that indicates the amount of used power in each subsystem. - Stored numerical value of average power usage and peak power usage for each subsystem that is stored in advance. - Function to measure the amount of remaining power - Communication with subsystems on power control command channel - Communication channel with emergency manager process - Status of power cut off switch |

  - Environment Sensing

| Testing Target | LADAR sensors |
| --- | --- |
| Objective | Tests the ability of the scanning LADAR sensors to sense the local environment |
| Function Verified | - Scanning LADAR sensors can each correctly sense the terrain elevation and ranges to obstacles near the lunar rover. - LADAR sensor accuracy/repeatability verified to function correctly through the compensation algorithm. - Obstacle detection verified to function correctly through each LADAR sensors |

| Testing Target | Stereo FLIR sensors |
| --- | --- |
| Objective | Tests the stereo FLIR sensors operability |
| Function Verified | - Stereo FLIR Subsystem (or system) function verified to be within calibration - Stereo FLIR sensors can sense the terrain elevation and thermal differences near the lunar rover - Obstacle detection verified to function correctly through the stereo FLIR |

## Processing

| Testing Target | Computation Server |
| --- | --- |
| Objective | This test indicates the operational capacity of computation resource |
| Function Verified | - Status of static/dynamic memory (database/processing) - Active CPUs (available processing power) - Active connection (communication & control to other sub-systems) |

## Navigation

| Testing Target | Pre-planning subsystem |
| --- | --- |
| Objective | Test the accuracy and effectiveness of the pre-planning subsystem |
| Function Verified | - Validate availability of pre-planning data - Validate continuity (completeness) of pre-planned data - Test processing speed (validate sample path calculation and loading speed) |

| Testing Target | Position Estimation & Navigation subsystem |
| --- | --- |
| Objective | Test the accuracy and effectiveness of the pre-planning subsystem |
| Function Verified | - Validate communication with location identification (Loran-c) system - Validate coordinate of the vehicle from Loran-c vs. preplanned data - Validate all navigation process are operational - Validate communication with sensor subsystem - Validate availability of space for additional data storage for calculated path - Validate communication with locomotion subsystem |