| 24-451 | Feedback Control Systems | Fall 2000 |
| --- | --- | --- |

Homework #1

Due Tuesday 12 September at 3 pm

- (5 points.) Use the defining integral of the Laplace transform and elementary properties of integrals to show the linearity of the Laplace transform, i.e., for functions f(t) and g(t) and real scalars a and b, one has

L[af(t)+bg(t)]=aL[f(t)]+bL[g(t)]

- (5 points.) Use the defining integral of the Laplace transform to prove the differentiation theorem of the Laplace transform, i.e., if F(s)=L[f(t)], then

L[f'(t)]=sF(s)−f(0+)

Hint: In general, given a function g(t) with a step discontinuity at t=0, one has

g(t)dt=g(t)dt+g(t)dt=g(t)dt=

- (10 points.) Use the defining integral of the Laplace transform to derive the Laplace transform of f(t)=eσtcosωt.

- (16 points.) Use the differentiation and time delay theorems of the Laplace transform to respectively show the following:

(a)  L[cosωt]=ss2+ω2

(b)  L[f(t)]=1s(1−e−sT),   where f(t) is given by the figure below:

# (24 points.) Assuming zero initial conditions, solve the following differential equations using the Laplace transform:

(a)  dxdt+7x=5cos2t

(b)  d2xdt2+6dxdt+8x=5sin3t

(c)  d2xdt2+8dxdt+25x=10u(t)

Use the MATLAB function residue to verify the partial fraction expansion in each case.