| 24-451 | Feedback Control Systems | Fall 2000 |
| --- | --- | --- |

Midterm Exam

- The figure below illustrates the automatic breaking control of a train, where Ur(s)=L[ur(t)], V(s)=L[v(t)], ur is the input voltage representing a desired train speed, and v is the actual train speed,. The amplifier gain is K=100, and the tachometer (velocity sensor) constant is Kg=0.15.

  - (10 points.) Suppose that the braking action can be modeled by a transfer function of the form GB=F(s)EB(s)=Cs+a, where EB(s)=L[eB(t)], F(s)=L[f(t)],  eB(t) is the braking input and f(t) is the resulting braking force. If f(t)=100(1-e-10t) is measured with eB=u(t) (the unit step function), determine the constants a and C.

  - (10 points.) The train has mass M=5104 and is assumed to be subjected to no other force than the braking force. What is the transfer function for the train, GT=V(s)F(s)?

  - (10 points.) Determine the closed-loop transfer function Tcl=V(s)Ur(s).

  - (10 points.) Let the input voltage be a step function ur(t)=Au(t). Determine the value of A if the steady state speed of the train (i.e., at time t  ) is to be maintained at 20. (Hint: use the Final Value Theorem of Laplace transforms.)

- Servomotor control systems often involve mechanical loads exhibiting significant elastic deformations. As shown in the figure below, the armature of a servomotor has resistance R with negligible inductance, torque constant K, and back emf constant Kb. (The field circuit for the motor is not shown.) The motor’s rotor, with moment of inertia Jm, is connected to a disc of inertia J by a shaft, which is equivalent to a torsional spring K.

  - (20 points.) First consider the mechanical component of this electromechanical system and denote by (t) the torque generated from the armature current. Derive the equations of motion for the motor’s rotor and the load disc (in either time domain or Laplace transform domain).

  - (20 points.) Let 1(s)=L[1(t)], 2(s)=L[2(t)], and T(s)=L[(t)]. The equations of motion (in the Laplace transform domain) of the rotor and the load disc have a solution of the form 1(s)=G1(s)T(s) and 2(s)=G2(s)T(s). Obtain an expression for G1(s) and G2(s).

  - (20 points.) Consider the system’s mechancal/electrical componenets together. Determine the transfer function G=Θ2(s)Ea(s), where Ea(s)=L[ea(t)], in terms of system parameters as well as G1(s) and G2(s).