| 24-451 | Feedback Control Systems | Fall 2000 |
| --- | --- | --- |

Quizz #1

- (20 points.) Use the differentiation theorem for Laplace transforms to derive the expression for F(s)=L[cosh(t)]. Note that the hyperbolic cosine and sine functions have the following properties:

cosh(0)=1, sinh(0)=0, ddtcosh(t)=sinh(t), and ddtsinh(t)=d2dt2cosh(t)=cosh(t).

- Consider the op-amp circuit shown below.

  - (20 points.) Show that the transfer function for the circuit can be written in the form

Vo(s)Vi(s)=KP+KIs+KDs,

and determine KP, KI and KD in terms of R1, R2, C1 and C2.

  - (10 points.) Can the transfer function Vo(s)Vi(s)=−s2+s+1s be implemented with this circuit with practically realizable resistance and capacitance values? Hint: look at the zeros of the transfer function.

- Consider the translational mechanical system shown below. A mass M=1 is supported on the frictionless surface of a cart. In addition, the mass is attached to a spring K=2 and a damper D=3, which in turn are attached to the cart.

  - (25 points.) Determine the transfer function from the cart displacement R(s) to the mass displacement Y(s). Both displacements are relative to the stationary ground.

  - (25 points.) From the transfer function, compute the time response y(t) for a unit step input r(t)=u(t) (assuming zero initial conditions).